Milestone Three Lab Questions

By: Stephen Lewis

1. Why does the loop that processes the LED blinking need to run in a separate thread?

The LED blinking loop needs to run in a separate thread because the Morse code transmission takes time to complete. The program has to wait through the dot, dash, and pause timings while the message’s being sent. If the work was handled by the main loop, the program could get tied up waiting for the current sequence to finish and wouldn’t be as responsive to the button. In the program, the transmit method runs in its own thread while the button event can still call processButton(). I saw this during testing. I could press the button while SOS was still being sent. The program registered the button press, finished the current message, and then changed to OK on the next pass.

2. What is the purpose of returning to the off state after each completed state action?

Returning to the off state gives the state machine a consistent starting point before it moves to the next action. The dot, dash, dot/dash pause, letter pause, and word pause states all transition from off and then return to off when they’re finished. It keeps one state from running directly into another and also makes sure an LED isn’t accidentally left on. It also makes the Morse code easier to follow.  Each flash or pause completes before the next state begins.

3. How could you integrate serial communications to facilitate changing the messages available to the program?

Serial communication could be added so another device could send a new message to the raspberry pi instead of having SOS and OK be the only choices written into the program. The pi could listen on a serial connection for incoming text. After receiving a complete message, the program could store it as one of the available messages and convert each character with the existing Morse code dictionary. I’d still have the program wait until the current transmission was finished before using the new message. It would keep the same behavior I tested with the push button and prevent a Morse code message from changing halfway through.

4. How could you use the 16x2 display to provide debugging information to the user when they don’t have access to the application console?

The 16x2 display could show short status messages in which normally appear in the terminal. My program already uses the display to show which message’s being sent, so I could expand that idea to show information like the current state or when the button has changed the active message. If something went wrong, the display could also show a short error or status message. It would be useful when the pi’s running by itself and I can’t see the SSH terminal or application console.